Slide Title & Introduction

Hello everyone. My name is Yusuf Tahir Orhan, and today I will present the implementation phase of my project: 'DeFT: A Deadlock-Free and Fault-Tolerant Routing Algorithm for 2.5D Chiplet Networks'. This study is not just a course project; it serves as the technical baseline for my Master's thesis, focusing on computer architecture and reliable system design."

Slide 1: Overview & Motivation

To understand why this is important, we must look at 2.5D integration. It is becoming the industry standard because it is highly scalable and cost-effective. But it introduces two critical challenges. First, as shown in the figure, inter-chiplet traffic patterns can create cyclic dependencies, leading to system-wide deadlocks. Second, the Vertical Links that connect these chiplets are physically fragile and prone to manufacturing or thermal defects. In modular systems, a single link failure can lead to complete system collapse.

Slide 2: Problem Definition

For this implementation, I am modeling a system architecture with 4 CPU chiplets integrated on an active interposer. Each chiplet consists of a 4x4 mesh network with 4 bidirectional vertical links. My primary objectives are to guarantee 100% deadlock-freedom and to maintain full network reachability even in severe scenarios where up to 25% of the vertical links fail. To test this, I will use both synthetic traffic patterns and real-world application traces from the PARSEC benchmarks.

Slide 3: Related Work & Project Positioning

I want to clearly position this work: it is a replication study of the DeFT algorithm. While existing methods like MTR or RC solve deadlocks, they often limit path redundancy, making them poor at handling link failures. My project establishes a high-fidelity, cycle-accurate hardware baseline. Once this foundation is solid, my Master's thesis will focus on extending this framework with reinforcement learning to optimize routing decisions dynamically.

Slide 4: Methodology - System Architecture & MoC

I am extending the Noxim simulator, which is a C++ based, cycle-accurate environment. In terms of Model of Computation, the project uses a Discrete-Event model for packet generation and fault injection. Simultaneously, the internal router mechanics follow a Synchronous Reactive model, where every router follows a strictly timed 5-stage synchronous pipeline from routing to link traversal.

Slide 5: Methodology - Implementation Details

The implementation covers two core logic areas. First is the deadlock avoidance logic, which relies on three strict Virtual Network assignment rules to break all cyclic dependencies in the 'up-down-up' traffic flow. Second is the fault-tolerance mechanism. Instead of real-time calculations that add latency, I am implementing offline-generated Lookup Tables. These tables allow boundary routers to dynamically redirect traffic to functional links based on the current fault state of the network."

Slide 6: Evaluation Plan

My evaluation plan tracks three key metrics: Reachability, Average Latency, and Network Throughput. I will compare the DeFT implementation against standard XY routing baselines in both fault-free and fault-injected scenarios. The expected outcome is to maintain 100% reachability even at an 8-link fault rate, where standard algorithms would typically fail to deliver a significant portion of the traffic."

Slide 7: Embedded System Design (ESD) Considerations

Finally, we must consider the hardware constraints of such a system. To keep the area and power overhead below 2%, I am strictly limiting the design to exactly two Virtual Channels per port. We also fix the buffer depth to 4 flits to reflect realistic resource-constrained environments. The cycle-accurate nature of this simulation is vital here, as it allows us to analyze predictable timing bounds and ensure the reliability of the system under thermal stress